- Proje Adı: Akıllı Geçiş Kontrol Terminali

- Kullanıcı: Son Kullanıcı

- Amacı: Cihazın 2 farklı çalışma modu bulunuyor. Access Controller Mode ve Reader Mode. 
Access Controller Mode: İnputlarına göre Access control yapıyor. Log tutuyor. Ve kapı kilidini açıyor. 
Reader Mode: İnputlarından aldığı UID’leri Wiegand-Output 26-34bit ya da RS485-Private-OSDP aracılığıyla Ana Acces Kontrolcüye gönderir.

- Authentication Type:

  - Mifare M1 Card

  - Touch PIN

  - QR Code

  - External Card Reader, QR Reader with Wiegand- Input(26-34bit) or RS485(Private-OSDP)

  - External Button (Digital Input)

- Verification Type:

  - Uzak Sunucu (Wi-Fi)

  - Mobile App (Bluetooth)

  - Main Access Controller with RS485- OSDP protocol

- Hardware Specs:

  - Power Supply : 12V DC

  - Components

    - MCU: ESP32-WROOM-32E-N4

    - RFID: MFRC522 IC

    - Touch: TSM12 IC

    - QR: XR1300 Module

    - RS485: THVD2410DR

    - Shift Register: SN74HC595

    - Buzzer: with Circuit

    - Relay

    - Wiegand Interface: Input- Output

    - Digital Inputs: 2 inputs

  - MCU Pin Configuration:

    - RFID:

      - MOSI: GPIO 5 (Requires external pull-up resistors, e.g., 4.7kΩ)

      - MISO: GPIO 36

      - CLK: GPIO 15 (Requires external pull-up resistors, e.g., 4.7kΩ)

      - CS: GPIO 0 (Requires external pull-up resistors, e.g., 4.7kΩ)

      - IRQ: GPIO 35 (Used for interrupt-driven scanning to free up the CPU)

      - RST: 74HC595-2-QG  (Note: MFRC522 operates at 3.3V)

    - Touch PIN (TSM12):

      - SDA: GPIO 32 (Requires external pull-up resistors, e.g., 4.7kΩ)

      - SCL: GPIO 33 (Requires external pull-up resistors, e.g., 4.7kΩ)

      - INT: GPIO 14 (Interrupt output from TSM12, very important for FreeRTOS task notification)

      - OUT: GPIO 27

      - RST: 74HC595-2-QH

    - QR Code Reader (XR1300):

      - TX: GPIO 25 (Connect to QR Reader RX)

      - RX: GPIO 39 (Connect to QR Reader TX)

    - Shift Register Digital (SN74HC595):

      - CLK (SHCP): GPIO 4

      - DATA (DS): GPIO 16

      - LATCH 1 (STCP for First Block): GPIO 17

      - LATCH 2 (STCP for Second Block): GPIO 13

    - Wiegand Interface :

      - D0 (Data 0): GPIO 22 (Input/Output capable logic)

      - D1 (Data 1): GPIO 23 (Input/Output capable logic)

    - RS485 / OSDP Interface:

      - RX (RO): GPIO 34 (Input only pin)

      - TX (DI): GPIO 26

      - DE/RE (Data/Receive Enable): GPIO 18

    - Digital Input (e.g., Door contact, Exit Request button):

      - Input-1: GPIO 19

      - Input-2: GPIO 21

    - Buzzer:

      - PWM Output: GPIO 12

    - Relay:

      - Relay-EN: GPIO 2

    - Shift Register-Outputs (SN74HC595 x2):

      - SN74HC595-1

        - QA: LED_EN_1

        - QB: LED_EN_2

        - QC: LED_EN_3

        - QD: LED_EN_4

        - QE: LED_EN_5

        - QF: LED_EN_6

        - QG: LED_EN_7

        - QH: LED_EN_8

      - SN74HC595-2

        - QA: LED_EN_9

        - QB: LED_EN_10

        - QC: LED_EN_11

        - QD: LED_EN_12

        - QE: STATUS_LED_EN

        - QF: Wiegand Output Enable

        - QG: RFID_RST

        - QH: TSM_RST

- Configurations via Mobile App

  - Set Wi-Fi Parameters

  - Set Door Parameters

    - Set Open Duration : Configurable Time: 1 to 255 seconds.

  - Set Exit Button Parameters

    - Exit Button Enable/Disable

  - Set Working Mode:

    - Access Control Mode:

    - Reader Mode:

  - Set Wiegand Parameters

    - Wiegand Enable/ Disable

    - Wiegand Direction

      - Input/Output

    - Wiegand Mode

      - Wiegand26

      - Wiegand34

    - Wiegand Time Interval

      - 1ms to 100ms

    - Wiegand Pulse Width

      - 10us to 1000us

  - Set RS485 Parameters

    - RS485 Enable/Disable

    - RS485 Peripheral Type

      - Card Reader

      - Extension Module

      - Access Controller

    - RS485 Protocol

      - Private

      - OSDP

    - RS485 Address

      - Set the RS-485 Address according to your actual needs.

    - RS485 Baut Rate

      - The baud rate when the devices are communicating via the RS-485 protocol.

    - RS485 Data Bit

      - The data bit when the devices are communicating via the RS-485 protocol.

    - RS485 Stop Bit

      - The stop bit when the devices are communicating via the RS-485 protocol.

- Acces Control Settings via Card

  - Full Installation Settings

  - Admin Installation Setting

  - User Access List Settings

  - Time Sync Settings

  - Factory Reset

- Access Control Settings via Remote Server

  - Set Door Parameters

    - Set Open Duration : Configurable Time: 1 to 255 seconds.

  - Set Exit Button Parameters

    - Exit Button Enable/Disable

  - Set Working Mode:

    - Access Control Mode:

    - Reader Mode:

  - Set Wiegand Parameters

    - Wiegand Enable/ Disable

    - Wiegand Direction

      - Input/Output

    - Wiegand Mode

      - Wiegand26

      - Wiegand34

    - Wiegand Time Interval

      - 1ms to 100ms

    - Wiegand Pulse Width

      - 10us to 1000us

  - Set RS485 Parameters

    - RS485 Enable/Disable

    - RS485 Peripheral Type

      - Card Reader

      - Extension Module

      - Access Controller

    - RS485 Protocol

      - Private

      - OSDP

    - RS485 Address

      - Set the RS-485 Address according to your actual needs.

    - RS485 Baut Rate

      - The baud rate when the devices are communicating via the RS-485 protocol.

    - RS485 Data Bit

      - The data bit when the devices are communicating via the RS-485 protocol.

    - RS485 Stop Bit

      - The stop bit when the devices are communicating via the RS-485 protocol.

    - Event Log

    - Factory Reset

      - Configuration – Tam Kurulum

Fabrika çıkışı boş bir kilide Cihaz ID, Master Key ve Sıfırlama Şifresi (AuthCode) NFC/Mifare kart aracılığıyla tek seferde yüklenir.

      - User List Update with Remote Server
“Ek_Onloi_Remote_Server_Protocol_V1.0” ek dosyasında belirtilmiştir.

      - Configuration – Tam Kurulum

Fabrika çıkışı boş bir kilide Cihaz ID, Master Key ve Sıfırlama Şifresi (AuthCode) NFC/Mifare kart aracılığıyla tek seferde yüklenir.

- User List Update with Card Reader
“Ek_Onloi_Access_Control_Protocol_V1.0” ek dosyasında belirtilmiştir.