Project: Wired and Wireless Smart Bluetooth Gateway

Description: 
Bu proje, piyasada bulunan ve yalnızca yakın mesafe haberleşme (Bluetooth Low Energy - BLE) yeteneğine sahip akıllı ev cihazlarını (sensörler, kilitler, aydınlatma ürünleri vb.) global internet ağına entegre etmeyi amaçlayan evrensel bir ağ geçidi (Gateway) çözümüdür. Sistem, marka bağımsız BLE uç cihazlarını tek bir merkezde toplayarak, bu cihazların Google Home ve Amazon Alexa gibi popüler sesli asistanlar ve bulut tabanlı otomasyon senaryoları üzerinden yönetilebilmesini sağlar.

Temel Çalışma Prensibi: 
Geliştirilen Gateway donanımı, sistem mimarisinde "merkezi köprü" (Central Bridge) rolü üstlenmektedir. Cihaz, kendi üzerinde herhangi bir karar mekanizması veya yerel otomasyon (Local Linkage) barındırmaz. Temel işlevi, çift yönlü ve kesintisiz bir veri çevirmenliği yapmaktır:

- Veri Toplama: Çevredeki BLE uç cihazlarından (örneğin bir sıcaklık sensöründen) gelen verileri GATT protokolü üzerinden okur.

- Buluta Aktarım: Toplanan bu lokal verileri, Wi-Fi üzerinden MQTT protokolünü kullanarak uzak sunucuya (Cloud) iletir.

- Aksiyon Yönetimi: Bulut sunucusunda çalışan otomasyon algoritmaları veya kullanıcının sesli asistan (Alexa/Google Home) komutları doğrultusunda üretilen kararları alır ve ilgili BLE uç cihazına (örneğin akıllı bir prize) komut olarak geri iletir.

FAZ1: Bluetooth
FAZ2: Zigbee 3.0
FAZ3: Multimode – Zigbee 3.0 & BLE & SigMesh

MCU: ESP32-WROOM-32E-N4
Internet Connection: Ethernet & Wi-Fi
Ethernet Transciever: LAN8720A-CP-TR
Ethernet Jack: HR911105A
Wifi haberleşme desteği: 802.11 b/g/n/e/i(802.11n, speeds up to 150 Mbps)

Wifi frekans: 2.4~2.5 GH

- Led Indicator:

Multi-Mode Gateway
Blue LED: Zigbee/Bluetooth Status Indicator
Red Led: Wi-Fi Status Indıcator

Bluetooth
Blue LED: Bluetooth Status Indıcator
Red LED: Wi-Fi Status Indicator

Connections:
USB Type-C: Power 5V-1A
Switch: Switch/Reset Key

- Bluetooth Performance

  - Bluetooth version: 
Bluetooth 5.0 low energy
(support 1M/2M/Coded LE PHY)

  - Bluetooth Roles: 
Oberserver role for scanning
Central role for connecting

  - Bluetooth scan PHY: 
Legacy BLE 4.2 1M PHY
Bluetooth 5 1M PHY
Bluetooth 5 2M PHY
Bluetooth 5 Coded PHY (long range)

  - Active scanning: 
Request scan response packets (By default)
Can be disabled (passive scanning)

  - Filter Beacon Protocols: 
Apple iBeacon
Google Eddystone – UID
Google Eddystone – URL
Google Eddystone – TLM
Onloi Beacon –

  - Beacon Data Format: 
JSON array format
Hexadecimal format

- Bluetooth Smarts – Connect

  - Bluetooth Connection PHY:
Automatic (1M/2M/Coded PHY)

  - Connection BLE peripheral:

Gateway connects to a BLE peripheral and sets up a communication process between the BLE peripheral and the Server. 
Only MQTT supports data transfer between the BLE peripheral and Server (Bluetooth Communication topic).

- Network

  - Internet Connection: 
WLAN to Internet
Ethernet to Internet
Network failover

  - Server Access Protocols: 
MQTT (SSL/TLS, socks5 proxy)
HTTP (SSL/TLS)
TCP
UDP

- Device Management

  - Web GUI: 
A Web user interface for configuring the gateway.
User can access the Web GUI via Wi-Fi (AP mode) or Ethernet (WAN IP).

  - Cloud Management: 
Monitoring and configuring the gateway remotely by Cloud management commands. 
Only MQTT supports (Remote Management topic)

  - System Logs: 
Record the important operations and events when the gateway Works

  - System Monitor: 
Gateway Running time
CPU temperature
CPU Usage
Memory Usage
Network/Server Connection Status

  - Time: 
ISO 8601 (JSON Format)
Unix Time (Hex Format)
Sync System time with NTP Server(s)

  - Restart:
Schedule the gateway to restart automatically at a certain time

Ecosystem: